My own pictures didn’t come out too well, so I’ve borrowed these from the UK owner’s club site (sorry).

This is a 2-man job and assuming no major problems will take about 2-3 hours. I must stress that this is advice only from me having done this job. If you have any doubts about being able to do this job, DON’T ATTEMPT IT. I accept no responsibility if you damage your car.

- Remove the battery and air inlet pipe.

- Undo the 2 bolts holding the throttle cable and earth straps (blue arrows).

- Remove the small rubber pipe from the top of the throttle body (green arrow).

- Remove the vacuum hose coming from the servo (remove at the manifold).

- Next remove the throttle body by undoing the 4 12mm bolts holding it in place (blue arrows below).

- Then remove the vacuum hose coming from the fuel pressure regulator.

- At this stage remove the front cambelt cover (10mm bolts) and remove the bolts from the rear cover also.

- The rear cover will not come out without removing the alternator belt tensioner but this is not necessary, so just leave the rear cover hanging in place with the bolts removed.

- To remove the manifold there are 3 bolts at the back (12mm) which need to come off.

- There are 7 bolts and 2 nuts at the front of the manifold that come off next. These are all different lengths so make sure to lay them out in the order they come out to get them back in the right places.

- The 2 outside nuts above can be a bit tricky to get out because they have a tendency to fall. Be very careful with these!!!!!

- The inlet manifold can now be removed and placed aside. Cover the exposed holes with a clean rag. During the rest of the removal process you may be tempted to place bolts and tools on this rag, DON’T. If the rag moves they fall into somewhere you really don’t want them to go.

- The next step is to remove the front coil cover and place aside (blue arrows below).

- When I was doing this I didn’t have to do step 13 because it was only the rear bank I had to work on. The process is the same for both front and rear regardless.

- Mark each spark plug wire so you’ll know what order they go back in. It’s probably safest to work on 1 side at a time so leave them in place on the side you’re not working on and then replace them before starting the opposite side.

- By doing it this way the wires are only long enough to go into the correct plug. If the wires are connected into the wrong plug, the car won’t start so take your time on this one.

- On the front bank remove the coils (10mm bolts) and take out of the way. This is the point where you could change your front spark plugs if required.

- With the coils removed there are 9 more 10mm bolts to take off the cover. These will not be very tight so don’t put them back on too tight, although if you regularly have to top up your oil like I did, it’s probably leaking from here so go a little tighter but not too far.

- The rear bank is the same whereby there are 9 bolts to be removed. The covers will be a little tight so gently pry them with a large flat head screwdriver, taking care not to damage the gaskets because you might need them again.

- With the covers off you can now see the exposed camshafts, followers, springs and lifters.

- At this stage you need a valve spring compressor, which I didn’t have so we modified a piece of 2” x 1” timber to keep the valve spring compressed. This is where the 2nd person comes in handy because without the valve spring compressor you need somebody constantly minding the valve spring. If this jumps up then you will probably have to take out the camshaft to sit everything back down properly.

- Place the car in neutral and with a half inch socket extension placed on the crankshaft pulley you can turn the engine to move the camshaft to get the lobe out of the way to release the follower.

- Take out the follower and then the lifter can be removed.

- The follower and lifter can be seen below (lifter on the right).

- Using vernier calipers, compare the length and diameter of the new and old lifter to ensure the right one is going in.

- Once the lifter is in (a thin film of oil on this first does no harm), carefully replace the follower into place. The follower and lifter go together like a loose ball and socket joint.

- The picture below shows the camshaft with the lifter and follower removed. The lifter slots into the sleeve marked in blue and the follower spreads from the lifter across under the camshaft lobe.

- The whole removal process is relatively easy but once the camshafts are exposed then you either need to have some experience in this area or have somebody with you who does.

- Once the lifters have been changed and the followers are back in have your assistant rotate the engine from the crankshaft pulley for a couple of revolutions and watch the camshafts to ensure everything is working as it should.

- Replacing everything is the reverse of removal and this is where memory comes into play. The bolts really need to go back into the same spots they came out of, especially on the manifold itself as they are different lengths.

- Once everything is in place and the correct tightness, remove any stray tools from the engine area. Start the car and listen to her purr.